Explainable, motion-tolerant super-resolution reconstruction 
for very low-field MRI

- Approach: This work will focus on mitigating the challenge of low spatial resolution at very low field MRI by combining up to three orthogonal views in a manner that generalizes across contrasts and is explainable by visualizing interpolations. This will be accomplished by a deep-learning network that will perform three tasks: (i) denoising using sparse transforms, ii) estimating motion field maps and mitigating for rotation and translation effects between the views, iii) combining the views at multiple scales to provide a super-resolved reconstruction (SRR) that does not suffer from blurring or hallucinations.

Novelty:  To the best of our knowledge, based on published literature, the state-of-the-art low-field SRR works1–3 (including the manuscripts at https://doi.org/10.1101/2024.02.16.580639,   https://doi.org/10.48550/arXiv.2202.03564)  have yet to focus on explainability or encapsulating the pipeline challenges of very low-field MRI, especially motion between the three views. The following is a list of novel features of our prospective implementation.

  - Explainability

  - Multi-scale SRR

  - Transform denoising and reconstruction

  - Motion tolerant to translation and rotation

  - Faster than existing analytical solutions such as the Niftymic4

- Generator (Training data): We will exclusively train on 3D data acquired at 3T and use the low-field data for our validation studies. Our previous denoising 5–7 and motion modeling 8,9 capabilities will help corrupt images.

- Network: We will use a combination of residual networks and U-net for the SRR task in a cascaded manner that exploits multi-scale implementations.

- Testing: The paired 3T and 0.064T data will serve as test data quantified for structural similarity index measure, peak signal-to-noise ratio, and brain volumetry. Any differences in contrast between 3T and 0.064T data will be addressed by computation of normalized mutual information.